Wire connection

I'll create a circuit diagram (schematic) showing the wiring connections of all your components with the ESP32, including:

- MQ-7 (CO Gas Sensor)

- MQ-138 (Air Quality Sensor)

- GPS Module (NEO-6M)

- BMP180 (Pressure Sensor)

- Rain Detection Sensor

- DHT22 (Temperature & Humidity Sensor)

- Ultrasonic Sensor (HC-SR04) for Water Level Measurement

Here’s the circuit wiring plan for your ESP32 Disaster Monitoring System with all sensor connections.

## 🛠 Wiring Guide

### ESP32 Pin Connections:

| Component | Type | ESP32 Pin |
| --- | --- | --- |
| MQ-7 (CO Gas Sensor) | Analog | GPIO 36 (VP) |
| MQ-138 (Air Quality Sensor) | Analog | GPIO 39 (VN) |
| GPS Module (NEO-6M) | UART (TX/RX ) | TX → GPIO 16, RX → GPIO 17 |
| BMP180 (Pressure Sensor) | I2C (SDA/SCL) | SDA → GPIO 21, SCL → GPIO 22 |
| Rain Detection Sensor | Digital | GPIO 5 |
| DHT22 (Temperature & Humidity) | Digital | GPIO 4 |
| Ultrasonic Sensor (HC-SR04) | Digital | Trig → GPIO 12, Echo → GPIO 14 |