## 1️⃣ Updated Components & Pin Configuration

| Component | Type | ESP32 Pin |
| --- | --- | --- |
| MQ-7 (CO Gas Sensor) | Analog | GPIO 36 |
| MQ-138 (Air Quality Sensor) | Analog | GPIO 39 |
| GPS Module (NEO-6M) | UART (TX/RX) | TX → GPIO 16, RX → GPIO 17 |
| BMP180 (Pressure Sensor) | I2C (SDA/SCL) | SDA → GPIO 21, SCL → GPIO 22 |
| Rain Detection Sensor | Digital | GPIO 5 |
| DHT22 (Temperature & Humidity) | Digital | GPIO 4 |
| Ultrasonic Sensor (HC-SR04) | Digital | Trig → GPIO 12, Echo → GPIO 14 |

## 2️⃣ Circuit Diagram (Schematic)

I'll generate the wiring diagram and share it soon.

## 3️⃣ Updated Arduino Code for ESP32

### 🔹 Required Libraries

In Arduino IDE, go to Sketch → Include Library → Manage Libraries and install:
✅ FirebaseESP32.h (For Firebase data upload)
✅ Wire.h (For BMP180 pressure sensor)
✅ Adafruit_BMP085.h (For pressure readings)
✅ DHT.h (For temperature & humidity)
✅ TinyGPS++.h (For GPS tracking)

#include <WiFi.h>

#include <FirebaseESP32.h>

#include <Wire.h>

#include <Adafruit_BMP085.h>

#include <DHT.h>

#include <TinyGPS++.h>

// WiFi Credentials

#define WIFI_SSID "your_wifi_name"

#define WIFI_PASSWORD "your_wifi_password"

#define API_KEY "your_firebase_api_key"

#define DATABASE_URL "your_firebase_database_url"

FirebaseData fbData;

FirebaseAuth auth;

FirebaseConfig config;

// Sensor Pins

#define MQ7_PIN 36       // MQ-7 CO Sensor

#define MQ138_PIN 39     // MQ-138 Air Quality Sensor

#define RAIN_SENSOR 5    // Rain Detection

#define DHTPIN 4         // DHT11/DHT22 Sensor

#define DHTTYPE DHT22    // Change to DHT11 if using DHT11

#define TRIG_PIN 12      // Ultrasonic Sensor Trig

#define ECHO_PIN 14      // Ultrasonic Sensor Echo

// GPS Module

#define RX_PIN 16

#define TX_PIN 17

DHT dht(DHTPIN, DHTTYPE);

Adafruit_BMP085 bmp;

TinyGPSPlus gps;

HardwareSerial gpsSerial(1);  // Use Serial1 for GPS

void setup() {

Serial.begin(115200);

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

gpsSerial.begin(9600, SERIAL_8N1, RX_PIN, TX_PIN);

dht.begin();

bmp.begin();

pinMode(MQ7_PIN, INPUT);

pinMode(MQ138_PIN, INPUT);

pinMode(RAIN_SENSOR, INPUT);

pinMode(TRIG_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

Serial.print("Connecting to WiFi");

while (WiFi.status() != WL_CONNECTED) {

Serial.print(".");

delay(1000);

}

Serial.println("\nConnected!");

config.api_key = API_KEY;

config.database_url = DATABASE_URL;

Firebase.begin(&config, &auth);

}

float measureWaterLevel() {

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

float duration = pulseIn(ECHO_PIN, HIGH);

float distance = (duration * 0.0343) / 2;  // Convert to cm

return distance;

}

void loop() {

float temperature = dht.readTemperature();

float humidity = dht.readHumidity();

float pressure = bmp.readPressure() / 100.0;  // Convert to hPa

int gasCO = analogRead(MQ7_PIN);

int airQuality = analogRead(MQ138_PIN);

int rain = digitalRead(RAIN_SENSOR);

float waterLevel = measureWaterLevel();

bool gpsAvailable = false;

float latitude, longitude;

while (gpsSerial.available() > 0) {

if (gps.encode(gpsSerial.read())) {

latitude = gps.location.lat();

longitude = gps.location.lng();

gpsAvailable = true;

}

}

// Send data to Firebase

if (Firebase.ready()) {

Firebase.setFloat(fbData, "/HydroSOS/Temperature", temperature);

Firebase.setFloat(fbData, "/HydroSOS/Humidity", humidity);

Firebase.setFloat(fbData, "/HydroSOS/Pressure", pressure);

Firebase.setInt(fbData, "/HydroSOS/CO_Level", gasCO);

Firebase.setInt(fbData, "/HydroSOS/AirQuality", airQuality);

Firebase.setInt(fbData, "/HydroSOS/Rain", rain);

Firebase.setFloat(fbData, "/HydroSOS/WaterLevel", waterLevel);

if (gpsAvailable) {

Firebase.setFloat(fbData, "/HydroSOS/Latitude", latitude);

Firebase.setFloat(fbData, "/HydroSOS/Longitude", longitude);

}

Serial.println("Data sent to Firebase");

}

delay(5000);

}

## 4️⃣ How It Works

✅ ESP32 collects sensor data every 5 seconds.
✅ Water level is measured using an ultrasonic sensor (If level is high, flood risk increases).
✅ Real-time data is sent to Firebase, which updates live on web & mobile apps.
✅ GPS logs location of potential disasters.

## 5️⃣ Web Dashboard for Live Monitoring

To view the data in real-time, create a Firebase Web Dashboard using HTML + JavaScript.

Do you want me to provide a Firebase web dashboard or mobile app code? 🚀

## 6️⃣ Next Steps

- 🎨 Circuit Diagram (I'll generate a proper schematic)

- 📲 Flutter Mobile App (For monitoring on Android/iOS)

- 🚀 AI-Based Prediction (Use ML for better disaster detection)

## How to Integrate AI/ML for Disaster Prediction

### 1️⃣ Collect & Store Weather Data

- Use ESP32 + Sensors (temperature, humidity, air quality, pressure, water level, etc.).

- Fetch real-time weather data from Google Weather API / OpenWeatherMap / NASA APIs.

- Store data in Firebase or a cloud database (like AWS, Google Cloud, or MongoDB).

### 2️⃣ Train an AI Model for Disaster Prediction

Use Machine Learning (ML) to detect patterns in weather conditions and predict disasters (floods, storms, pollution spikes).

#### ML Model Workflow

✅ Step 1: Gather historical weather data from Google Weather, NOAA, NASA.
✅ Step 2: Label data (e.g., "Flood," "Cyclone," "Heatwave," "Normal").
✅ Step 3: Train a Neural Network (LSTM, CNN) or Decision Tree to recognize patterns.
✅ Step 4: Deploy the model in Google Cloud ML, TensorFlow, or Edge AI on ESP32.
✅ Step 5: Predict disasters and send alerts based on AI analysis.

📌 Best ML Model for Weather Prediction:

- LSTM (Long Short-Term Memory): Works well with time-series data (e.g., temperature changes over time).

- CNN (Convolutional Neural Networks): Useful for satellite image analysis (cloud patterns, rainfall prediction).

- Random Forest / Decision Trees: Fast & efficient for classifying disaster risks.

### 3️⃣ Connect with Google Weather & Live Satellite Images

- Google Weather API → Get real-time temperature, wind speed, and storm updates.

- NASA Earthdata API → Fetch satellite images of cloud patterns.

- Google Earth Engine → Analyze climate trends using historical satellite data.

💡 How It Works:
📡 ESP32 collects local weather data → 🌍 AI model analyzes data + Google Weather → ⚠️ If risk detected, sends alerts to users & authorities.

## 4️⃣ Deploy AI Model for Real-Time Monitoring

### Option 1: Cloud-Based AI Model (Recommended for Real-Time)

- Train the AI model using Google Colab, TensorFlow, or PyTorch.

- Deploy it to Google Cloud ML, AWS SageMaker, or Firebase ML.

- ESP32 sends weather data to the cloud → AI processes it → Sends alerts.

### Option 2: Edge AI on ESP32 (For Offline Processing)

- Use TinyML (TensorFlow Lite) to run an AI model directly on ESP32.

- This can help in remote areas where the internet is unavailable.

## 5️⃣ Real-Time Dashboard for Monitoring

- Create a website or mobile app (Flutter, React) to visualize live weather trends, alerts, and satellite images.

- Integrate Google Maps API to show affected areas with disaster risk levels.

I'll create a circuit diagram (schematic) showing the wiring connections of all your components with the ESP32, including:

- MQ-7 (CO Gas Sensor)

- MQ-138 (Air Quality Sensor)

- GPS Module (NEO-6M)

- BMP180 (Pressure Sensor)

- Rain Detection Sensor

- DHT22 (Temperature & Humidity Sensor)

- Ultrasonic Sensor (HC-SR04) for Water Level Measurement

I'll generate the wiring diagram and share it shortly. Stay tuned! 🎨🔧🚀

Here’s the circuit wiring plan for your ESP32 Disaster Monitoring System with all sensor connections.

## 🛠 Wiring Guide

### ESP32 Pin Connections:

| Component | Type | ESP32 Pin |
| --- | --- | --- |
| MQ-7 (CO Gas Sensor) | Analog | GPIO 36 (VP) |
| MQ-138 (Air Quality Sensor) | Analog | GPIO 39 (VN) |
| GPS Module (NEO-6M) | UART (TX/RX) | TX → GPIO 16, RX → GPIO 17 |
| BMP180 (Pressure Sensor) | I2C (SDA/SCL) | SDA → GPIO 21, SCL → GPIO 22 |
| Rain Detection Sensor | Digital | GPIO 5 |
| DHT22 (Temperature & Humidity) | Digital | GPIO 4 |
| Ultrasonic Sensor (HC-SR04) | Digital | Trig → GPIO 12, Echo → GPIO 14 |